WARRANTY STANDARDS GUIDE

Performance Standards for Your Virginia Home

Based on the OHBA Minimum Quantifiable Standards, Seventh Edition

The Paragon Building Group, Ltd.

dba Virginia Homes

565 Metro Place South, Suite 220  |  Dublin, Ohio 43017

(614) 764-1953  |  www.virginia-homes.com

Table of Contents

| # | Section | Page |
| --- | --- | --- |
| 1 | Introduction | 3 |
| 1a | How to Use This Guide | 3 |
| 1b | Priority of Standards | 4 |
| 2 | Your Warranty at a Glance | 5 |
| 2a | One-Year Limited Warranty | 5 |
| 2b | Ten-Year Structural Warranty | 5 |
| 2c | Exclusions from Coverage | 6 |
| 2d | Manufacturer Warranties | 7 |
| 3 | Site Work | 8 |
| 4 | Foundation | 9 |
| 5 | Interior Floor Construction | 11 |
| 6 | Walls & Framing | 12 |
| 7 | Exterior Finish | 13 |
| 8 | Roofing & Gutters | 14 |
| 9 | Plumbing | 15 |
| 10 | Electrical | 16 |
| 11 | Interior Climate Control (HVAC) | 17 |
| 12 | Interior Finish | 18 |
| 13 | Miscellaneous | 22 |
| 14 | Exclusions & Limitations | 23 |
| 15 | How to Submit a Warranty Claim | 24 |
| 16 | Homeowner Maintenance Responsibilities | 25 |
| 17 | Contact Information | 26 |
| A | Quick-Reference Tolerance Table | 27 |

INTRODUCTION

The Warranty Standards Guide outlines the standards and tolerances that Virginia Homes uses to ensure quality construction and customer satisfaction. This guide is based on the OHBA (Ohio Homebuilders Association) Minimum Quantifiable Standards, Seventh Edition, and represents Virginia Homes&#x2019; commitment to building homes that meet or exceed industry standards.

Warranty coverage is provided by Virginia Homes and is subject to the terms and conditions outlined in your Warranty Certificate. All standards outlined in this guide apply during the applicable warranty period.

How to Use This Guide

This guide is organized by construction category. Each section includes specific standards for workmanship and materials. Standards are presented as tolerances or ranges that are acceptable for your home. If you believe your home does not meet these standards, please refer to the &#x201c;How to Submit a Warranty Claim&#x201d; section for instructions.

Priority of Standards

| Priority Level | Standard Source | Application |
| --- | --- | --- |
| 1 | RCO (Residential Code of Ohio) | Virginia Homes follows RCO codes and regulations |
| 2 | Manufacturer Instructions | Material and appliance manufacturer specifications apply |
| 3 | OHBA Standards | Minimum Quantifiable Standards guide workmanship |

YOUR WARRANTY AT A GLANCE

One-Year Limited Warranty

Virginia Homes provides a one-year limited warranty on all construction workmanship and materials. This warranty covers defects in workmanship and materials that do not conform to the standards outlined in this guide. The warranty period begins on the date of closing and extends for one year.

Ten-Year Structural Warranty

Virginia Homes provides a ten-year structural warranty that covers the structural integrity of the home. This warranty protects against major structural defects that would affect the safety or habitability of the home. The structural warranty does not cover cosmetic issues, settling, or normal wear and tear.

Exclusions from Coverage

The following items are excluded from warranty coverage:

- Normal settling and cracking (hairline cracks less than 1/8 inch)

- Cosmetic variations in materials and finishes

- Damage caused by homeowner misuse or neglect

- Damage caused by natural disasters or acts of God

- Modifications made by homeowner without Virginia Homes approval

- Damage from improper maintenance or failure to follow homeowner maintenance responsibilities

- Manufacturer defects (covered by manufacturer warranty)

- Damage from pest infestation or animal damage

- Condensation or moisture issues caused by homeowner use

- Damage from trees, landscaping, or site conditions

- Issues caused by failure to maintain proper ventilation or humidity levels

Manufacturer Warranties

Many products and appliances installed in your home come with manufacturer warranties. These warranties are separate from Virginia Homes warranty and are provided directly by the manufacturer. Manufacturer warranties may offer longer coverage periods or different terms than Virginia Homes warranty. Refer to the warranty documentation provided for specific products for details.

SITE WORK

| Component | Standard | Tolerance |
| --- | --- | --- |
| Grading | Lot shall be graded to slope away from home foundation at minimum 1% grade | 1-2% slope |
| Swales & Drainage | Swales and drainage shall direct water away from foundation | Minimum 1% slope |

Not Covered

Changes to grading or drainage after closing are the homeowner&#x2019;s responsibility. Erosion and settling of landscaping materials are not covered by warranty.

FOUNDATION

General Foundation Standards

| Component | Standard | Tolerance |
| --- | --- | --- |
| Foundation Walls | Foundation walls shall be plumb and stable | 1/2 inch in 10 feet |
| Basement Floors | Basement floors shall be level and free from major cracks | 1/2 inch in 10 feet |

Concrete Slabs

| Component | Standard | Tolerance |
| --- | --- | --- |
| Slab Flatness | Slabs shall be reasonably flat for flooring application | 1/4 inch in 10 feet |
| Slab Cracks | Control joints crack at intended locations; structural cracks covered if exceeding standards | 1/8 inch width |

Basement & Crawl Space Walls

| Component | Standard | Tolerance |
| --- | --- | --- |
| Wall Cracks | Minor cracks acceptable; structural cracks repaired | Hairline - 1/8 inch |
| Wall Bowing | Walls shall not bow inward excessively | Less than 1 inch deviation |
| Water Seepage | Reasonable efforts made to prevent water intrusion | Minor seepage acceptable |
| Floor Cracks | Minor cracking acceptable; major cracks repaired | 1/8 inch width |

Structural Columns

| Component | Standard | Tolerance |
| --- | --- | --- |
| Column Plumb | Columns shall be plumb and vertical | 1/4 inch in 8 feet |
| Column Base | Column base shall be secure and level | 1/2 inch variation |
| Support Beams | Beams shall be properly supported and secured | No movement allowed |
| Beam Level | Beams shall be level or properly sloped | 1/2 inch in 20 feet |

INTERIOR FLOOR CONSTRUCTION

Subfloors

| Component | Standard | Tolerance |
| --- | --- | --- |
| Subfloor Flatness | Subfloors shall be flat and level | 1/4 inch in 10 feet |
| Subfloor Fastening | Subfloors shall be properly fastened to joists | No squeaking allowed |

Finished Flooring

| Component | Standard | Tolerance |
| --- | --- | --- |
| Vinyl/Laminate | Flooring shall be installed level and without gaps | 1/8 inch expansion gap |
| Wood Flooring | Wood flooring shall be properly acclimated and installed | 1/2 inch movement acceptable |
| Carpet | Carpet shall be properly stretched and fastened | No visible wrinkles |

Not Covered

Wood floor movement and expansion due to humidity changes is normal and not covered. Carpet staining, wear, or fading is not covered. Laminate or vinyl flooring chips or damage from improper installation is the responsibility of the installer.

WALLS & FRAMING

Walls

| Component | Standard | Tolerance |
| --- | --- | --- |
| Wall Plumb | Walls shall be plumb and vertical | 1/2 inch in 10 feet |
| Wall Straight | Walls shall be reasonably straight | 1/4 inch in 10 feet |

Drywall

| Component | Standard | Tolerance |
| --- | --- | --- |
| Drywall Finish | Drywall finish shall be smooth and properly taped | Minor imperfections acceptable |
| Drywall Cracks | Minor settling cracks acceptable; major cracks repaired | Hairline - 1/8 inch |
| Joint Compound | Joints shall be properly sealed and sanded smooth | Slight ridge acceptable |

Exterior Doors

Exterior doors shall be hung plumb and square. Doors shall open and close smoothly with proper weatherstripping. Minor adjustments may be needed during the first year due to settling.

Not Covered

Nail pops and settlement cracks are normal in new construction. Minor drywall imperfections that do not affect the overall appearance or function are not covered. Tape lines visible due to lighting are not considered defects.

EXTERIOR FINISH

Masonry

| Component | Standard | Tolerance |
| --- | --- | --- |
| Bow in Brick | Brick shall not bow excessively | Less than 1/2 inch in 10 feet |
| Broken/Loose Brick | Brick shall be securely mortared | No loose or missing brick |
| Surface Cracks | Minor surface cracks acceptable in mortar | Hairline - 1/8 inch |
| Mortar Obstructions | Mortar shall be properly struck and cleaned | Minor smudges acceptable |
| Cleaning | Brick shall be cleaned of excess mortar | Professional cleaning acceptable |

ROOFING & GUTTERS

| Component | Standard | Tolerance |
| --- | --- | --- |
| Gutter Leaks | Gutters shall direct water without leaking | No water intrusion in soffits |
| Roof Drainage | Roof shall drain properly with good pitch | Minimum 1/4 inch per foot |
| Roof Covering | Roof shingles/tiles shall be securely fastened | No missing or loose shingles |

Not Covered

Algae growth or discoloration on roof shingles is not covered. Gutters require seasonal maintenance and cleaning which is homeowner responsibility. Roof damage from weather, falling trees, or improper maintenance is not covered.

PLUMBING

Virginia Homes follows the Residential Code of Ohio (RCO) for all plumbing installations. All plumbing fixtures are installed to manufacturer specifications. Your home features PEX plumbing with Moen fixtures for durability and reliability.

Water pressure should be between 40-80 PSI. Water temperature at fixtures should not exceed 120 degrees Fahrenheit for safety. All drains should flow without backing up.

Water heaters are set to 120 degrees Fahrenheit. Tankless water heaters may have a slight delay in hot water delivery, which is normal. Homeowners should understand their water heating system and maintain it according to manufacturer instructions.

Not Covered

Slow drains caused by homeowner misuse, debris, or hair clogs are not covered. Leaking fixtures due to improper use are not covered. Water damage caused by frozen pipes is covered only if the home was properly heated. Cosmetic issues with fixtures are not covered.

ELECTRICAL

Virginia Homes follows the RCO for all electrical installations. All electrical systems are inspected and approved by the city/township inspector. Your home features 200 AMP electrical service with modern breaker panels for adequate capacity.

GFCI (Ground Fault Circuit Interrupter) outlets are installed in all wet areas including kitchens and bathrooms for safety. AFCI (Arc Fault Circuit Interrupter) protection is provided in bedrooms. Smoke and CO2 detectors are installed on each level.

Electrical outlets and switches should function properly. Some minor variations in outlet height or alignment are normal. Homeowners should not overload circuits and should follow safe electrical practices.

Not Covered

Tripped breakers due to overloaded circuits are not covered. Appliance damage from electrical surge is not covered. Light fixtures and plug-in appliances are not covered unless they are built-in and provided by the builder.

INTERIOR CLIMATE CONTROL (HVAC)

Your Virginia Homes home features a high-efficiency Carrier heating and cooling system. The furnace operates at 96% efficiency and the air conditioning unit is rated at 16 SEER for energy efficiency. Your home includes a two-zone heating system for independent temperature control in different areas.

A whole-home humidifier is installed to maintain proper humidity levels during winter months. Proper humidity levels (30-50%) help with comfort and reduce static electricity.

Temperature differentials between rooms should not exceed 3-5 degrees Fahrenheit during normal operation. Variation up to 10 degrees in extreme conditions may occur. Airflow balance may be adjusted during the first year of occupancy as the system settles.

Homeowners should change filters monthly and maintain annual maintenance per the HVAC contractor&#x2019;s recommendations. Proper ductwork sealing and insulation is critical for system performance.

Not Covered

System noise during normal operation is expected. Temperature variations caused by improper maintenance or filter replacement are not covered. Damage from improper use or electrical surge is not covered. Upgrading system components is not covered by warranty.

INTERIOR FINISH

Doors & Windows

| Component | Standard | Tolerance |
| --- | --- | --- |
| Door Operation | Doors shall operate smoothly and latch properly | Minor adjustments acceptable |
| Door Finish | Door finish shall not have chips or damage | Professional repair acceptable |
| Window Operation | Windows shall operate smoothly and not stick | Minor adjustments acceptable |
| Weatherstripping | Weatherstripping shall seal properly | Minor air infiltration acceptable |
| Glass Quality | Glass shall be clean and free from defects | No bubbles or cracks |

Stairs

| Component | Standard | Tolerance |
| --- | --- | --- |
| Stair Rise/Run | Stairs shall have consistent rise and run | 1/4 inch variation maximum |
| Stair Squeak | Stairs shall be securely fastened | Some movement acceptable |
| Handrails | Handrails shall be secure and code-compliant | No movement when tested |

Trim & Moldings

| Component | Standard | Tolerance |
| --- | --- | --- |
| Trim Installation | Trim shall be installed level and straight | 1/4 inch over 8 feet |
| Trim Fastening | Trim shall be securely fastened | No nails or fasteners visible |
| Caulking | Caulk joints shall be smooth and sealed | Minor imperfections acceptable |
| Paint Coverage | Trim paint shall have even coverage | Two coats minimum |

Cabinets

| Component | Standard | Tolerance |
| --- | --- | --- |
| Cabinet Alignment | Cabinets shall be level and aligned | 1/8 inch door gap tolerance |
| Cabinet Finish | Cabinet finish shall be smooth and even | Minor variations acceptable |
| Door Operation | Cabinet doors shall open and close smoothly | Self-closing hinges may require adjustment |
| Drawer Operation | Drawers shall slide smoothly | Minor binding acceptable |
| Hardware | Hardware shall be securely fastened | All knobs and pulls tight |
| Interior Finish | Interior shall be clean and smooth | Minor imperfections acceptable |
| Sealing | Joints and seams shall be sealed | Caulk lines acceptable |

Countertops

| Component | Standard | Tolerance |
| --- | --- | --- |
| Countertop Level | Countertops shall be level and even | 1/8 inch in 10 feet |

Interior Paint

| Component | Standard | Tolerance |
| --- | --- | --- |
| Paint Coverage | Paint shall have even coverage and color | Two coats minimum |
| Paint Quality | Paint shall adhere well without peeling | Professional quality expected |
| Masking | Paint lines shall be clean and straight | Minor imperfections acceptable |
| Trim Coverage | All trim and woodwork shall be painted | Even coverage required |

Wallpaper

| Component | Standard | Tolerance |
| --- | --- | --- |
| Wallpaper Seams | Seams shall be straight and well-adhered | Minimal visibility acceptable |
| Wallpaper Bubbles | Wallpaper shall not have air bubbles | Minor bubbles acceptable |
| Wallpaper Edges | Edges shall be properly trimmed and sealed | No peeling or separation |
| Pattern Match | Pattern match shall be accurate | Minor variations acceptable |
| Adhesion | Wallpaper shall be securely adhered | No peeling corners |

MISCELLANEOUS

Fireplace & Gas

| Component | Standard | Tolerance |
| --- | --- | --- |
| Gas Line Installation | Gas lines shall be properly installed per code | Pressure test required |
| Fireplace Operation | Fireplace shall operate as designed | Manufacturer manual applies |
| Damper Operation | Damper shall operate smoothly | Minor adjustment acceptable |
| Chimney Clearance | Proper clearance from combustibles | Code compliant spacing required |
| Sealing | Fireplace shall be sealed properly | No air leaks into living space |

Garage Doors

| Component | Standard | Tolerance |
| --- | --- | --- |
| Door Operation | Door shall operate smoothly and balance properly | Minor adjustments acceptable |
| Door Seal | Door seals shall prevent drafts and water intrusion | Weather stripping present |
| Hardware | All hardware shall be secure and functional | Springs and hinges checked |

Driveways & Sidewalks

| Component | Standard | Tolerance |
| --- | --- | --- |
| Concrete Flatness | Concrete shall be reasonably flat for vehicles | 1/4 inch in 10 feet |
| Concrete Cracks | Hairline cracks acceptable; control joints provided | Hairline - 1/8 inch |
| Concrete Slope | Proper drainage slope away from home | Minimum 1/8 inch per foot |
| Sealing | Concrete sealing recommended for longevity | Homeowner responsibility |

EXCLUSIONS & LIMITATIONS

The following items and conditions are excluded from Virginia Homes warranty coverage:

- Normal settling and minor cracks (hairline to 1/8 inch) in drywall, concrete, and masonry

- Cosmetic variations in finishes, colors, and materials due to normal manufacturing tolerances

- Damage caused by homeowner abuse, misuse, or neglect

- Damage from natural disasters, earthquakes, floods, fires, or acts of God

- Modifications or alterations made by homeowner without Virginia Homes written approval

- Improper maintenance or failure to follow homeowner maintenance responsibilities

- Defects in manufacturer-supplied appliances (covered by manufacturer warranty)

- Damage from pest infestation, termites, rodents, or animal damage

- Condensation, moisture, or humidity issues caused by improper homeowner use or ventilation

- Damage from tree branches, landscaping, or site conditions not caused by builder

- Issues caused by failure to maintain proper ventilation or humidity levels in home

- Caulk color variations due to aging or sun exposure after one year

- Paint color variations due to aging, sun exposure, or monitor variation

- Nail pops and settlement cracks in drywall typical of new construction

- Minor squeaks in floors or stairs due to wood movement

- Plumbing or electrical issues caused by homeowner modifications

- HVAC system operation noise during normal operation

- Temperature variations during extreme weather conditions

- Water damage from frozen pipes if home not maintained above 55 degrees

- Wear and tear on flooring, fixtures, and appliances from normal use

- Damage from improper or excessive use of mechanical systems

- Issues related to soil conditions, grading, or drainage after closing

- Rust or corrosion on exterior metals due to salt air or environmental conditions

Sole Expressed Warranty Notice

Virginia Homes warranty is the sole expressed warranty provided. Virginia Homes makes no other warranties, express or implied, including any warranty of merchantability or fitness for a particular purpose. Some states do not allow the exclusion of implied warranties, so these exclusions may not apply to you.

HOW TO SUBMIT A WARRANTY CLAIM

If you believe your home has a defect covered by Virginia Homes warranty, please follow these steps:

1. Contact Virginia Homes Customer Service at (614) 764-1953 or visit www.virginia-homes.com

2. Provide your address, closing date, and description of the issue

3. Include photos of the defect if possible for documentation

4. Schedule an inspection with Virginia Homes service representative

5. Virginia Homes will assess the issue and determine if it is covered by warranty

6. If covered, Virginia Homes will coordinate repairs or remediation

7. Follow up to ensure repairs meet Virginia Homes standards

Right to Cure / Bar on Claims

Virginia Homes retains the right to repair or remedy any defect covered by warranty. Homeowners must provide Virginia Homes a reasonable opportunity to cure the defect before pursuing other remedies. Warranty claims must be reported within the applicable warranty period (one year for general coverage, ten years for structural defects) or the claim will be barred.

HOMEOWNER MAINTENANCE RESPONSIBILITIES

To preserve your warranty coverage and maintain your home properly, homeowners have important maintenance responsibilities. Failure to perform basic maintenance may void warranty coverage for related defects. Below is a guide to seasonal maintenance activities:

| Season | Maintenance Tasks | Frequency |
| --- | --- | --- |
| Spring | Clean gutters and downspouts; inspect roof; check caulking around windows and doors; inspect foundation for new cracks | Annually |
| Summer | Maintain proper humidity levels (30-50%); change HVAC filter; inspect exterior paint and stain; trim trees away from house | As needed |
| Fall | Clean gutters and downspouts; winterize exterior faucets; inspect weatherstripping; seal gaps; prepare for winter | Annually |
| Winter | Keep home heated to minimum 55 degrees; monitor for ice dams; check basement for moisture; maintain HVAC system | Throughout season |

Additional homeowner responsibilities include: replacing HVAC filters monthly; having HVAC system professionally serviced annually; cleaning or replacing range hoods; maintaining lawn and landscaping; inspecting attic and basement periodically; testing smoke and CO2 detectors monthly; and addressing water issues immediately.

CONTACT INFORMATION

| Item | Details |
| --- | --- |
| Company Name | The Paragon Building Group, Ltd. dba Virginia Homes |
| Address | 565 Metro Place South, Suite 220, Dublin, Ohio 43017 |
| Phone | (614) 764-1953 |
| Website | www.virginia-homes.com |
| Hours | Monday - Friday 9:00 AM - 5:00 PM EST |

Thank you for choosing Virginia Homes.

We&#x2019;re committed to your satisfaction and the quality of your new home.

APPENDIX A: QUICK-REFERENCE TOLERANCE TABLE

| Component | Tolerance/Standard | Section |
| --- | --- | --- |
| Grading Slope | Minimum 1% away from foundation | Site Work |
| Foundation Walls | 1/2 inch in 10 feet plumb | Foundation |
| Concrete Slab | 1/4 inch in 10 feet flatness | Foundation |
| Basement Cracks | Hairline to 1/8 inch | Foundation |
| Subfloor Flatness | 1/4 inch in 10 feet | Floor Construction |
| Flooring Expansion | 1/8 inch gap minimum | Floor Construction |
| Wall Plumb | 1/2 inch in 10 feet | Walls & Framing |
| Wall Straight | 1/4 inch in 10 feet | Walls & Framing |
| Drywall Cracks | Hairline to 1/8 inch | Walls & Framing |
| Nail Pops | Normal - not covered | Walls & Framing |
| Brick Bow | Less than 1/2 inch in 10 feet | Exterior Finish |
| Roof Pitch | Minimum 1/4 inch per foot | Roofing & Gutters |
| Door Gaps | 1/8 inch tolerance | Interior Finish |
| Stair Rise/Run | 1/4 inch variation maximum | Interior Finish |
| Cabinet Door Gap | 1/8 inch tolerance | Interior Finish |
| Countertop Level | 1/8 inch in 10 feet | Interior Finish |
| Driveway Flatness | 1/4 inch in 10 feet | Miscellaneous |
| Water Pressure | 40-80 PSI normal | Plumbing |
| Water Temp | 120 degrees Fahrenheit | Plumbing |
| Electrical Service | 200 AMP standard | Electrical |
| HVAC Differential | 3-5 degrees normal | HVAC |
| Window Operation | Smooth with minor adjustments | Interior Finish |
| Door Operation | Smooth with minor adjustments | Interior Finish |
| Paint Coverage | Two coats minimum | Interior Finish |
| Caulking | Smooth and sealed | Interior Finish |
| Trim Alignment | 1/4 inch over 8 feet | Interior Finish |
| Fireplace Damper | Smooth with minor adjustment | Miscellaneous |
| Garage Door | Balanced and smooth | Miscellaneous |
| Concrete Cracks | Hairline to 1/8 inch | Miscellaneous |